CHAPTER 52:  SEWERS

Public and Private Sewers

52.01	Purpose

52.02	Prohibition Against Discharge into the Sanitary Sewerage System

52.03	Applicability

52.04	Connections and Installations (formerly 52.01)

52.15	Definitions

52.19	Building Sewers (Sewer Laterals) and Connections

Building Sewers (Sewer Laterals)

52.51	Administrator Authority to Enforce

52.52	Sewer Laterals, Cleanouts and Connections

52.53	Sump Pump/Foundation Drain Tile Regulations

52.54	Inspection of Sewer Laterals

52.55	Repair of Sewer Laterals

52.56	Sewer Lateral Requirements Associated with Building Demolition

52.57	Waivers

52.58	Surcharges and Penalties

52.59	Severability and Validity

PUBLIC AND PRIVATE SEWERS

52.01 	 PURPOSE

The Buhl City Council finds that the discharge of water from roofs, downspouts, surfaces, groundwater, sump pumps, footing tiles, swimming pools, air conditioning systems, area drains, or other means of transmitting natural precipitation into the City sanitary sewerage system will, and has in the past, flooded and/or overloaded the sanitary sewerage system to such an extent as to cause significant and grave damage to the wastewater treatment facilities and sanitary sewer collection system.  Such damage is caused by the backup of sewage into the city-served lift station(s), private properties, and pressure damage to collection lines.  The Council, therefore, finds it essential, for the minimization of damage to property and to meet Minnesota Pollution Control Agency and Buhl City Code requirements, that the provisions of this ordinance be strictly enforced.

52.02  PROHIBITION AGAINST DISCHARGE INTO THE SANITARY SEWERAGE SYSTEM

No water from any roof, surface, ground water, sump pump, foundation drain/footing tile, swimming pool, or other natural precipitation shall discharge, or cause to be discharged, into the municipal sanitary sewer system due to inflow, infiltration, pumps, defective plumbing, a defective sewer service lateral or by any other means.

52.03  	APPLICABILITY

This ordinance shall apply to all water entering the sanitary sewer system unless explicitly exempted by the City.  The City and its representatives are authorized to administer, implement, and enforce the provisions of this ordinance.

52.15  	DEFINITIONS

- “BUILDING SEWER”.  The extension from the Building Drain to the public sewer or other place of disposal, also referred to herein as a house connection, service connection, or sewer lateral.

- “SEWER LATERAL” or “LATERAL”, when used herein, means a privately owned pipeline connecting a Building Drain to the public sewer.

52.19 	 BUILDING SEWERS (SEWER LATERALS) AND CONNECTIONS

(B) No unauthorized person shall uncover, make any connections with or opening into, use, alter, or disturb any public sewer or appurtenance thereof without first obtaining a written permit from the City Public Utility Foreman or their authorized representative.

(C)  (1)   There shall be two classes of Building Sewer permits:

(a) For residential and commercial services; and

(b) For service to establishments producing industrial wastes.

(F) 	A separate and independent sewer lateral is not required for accessory buildings.

BUILDING SEWERS (SEWER LATERALS)

52.51 ADMINSTRATOR AUTHORITY TO ENFORCE.

The City Administrator/Clerk-Treasurer or their designated representatives shall be charged with the administration of the sewerage system and the enforcement of the provisions of this subchapter.

52.52  SEWER LATERALS, CLEANOUTS AND CONNECTIONS.

(A)	All laterals from the building wall to the connection to the sewer main are the property of the owner of the connected building.  All property owners whose properties are connected to a sewer main or are otherwise connected to the city’s sewer system by sewer lateral shall at their own expense maintain the sewer lateral in a fully functioning condition and ensure the lateral is free of cracks, leaks, inflow or infiltration of extraneous water, root intrusion or open joints.  Property owners shall ensure that laterals drain freely to the sewer main without excessive sags that collect grease and sediment.

(B)	No person, firm or corporation shall break or cut into or connect to any sewer in any street, easement, or right-of-way in the city or under the control of the city without first securing a permit to do so from the City Engineer and/or the City Public Utility Foreman.  Prior to beginning work, detailed plans describing the work to be done shall be submitted to and approved by the City Public Utility Foreman and/or City Engineer or his or her designee.

(C)	Each property utilizing the city's sewer system shall have a separate lateral connected to the sewer main.  Notwithstanding the foregoing, branched or common laterals shall be permitted only in the following instances:

(1)	Where a lateral is maintained by a homeowner’s association or other entity that is party to a formal, recorded lateral maintenance agreement.

(2)	Where more than one occupied building or other structure is situated upon the same lot, in which case all buildings and structures may, by permit authorized by the City Engineer and/or City Public Utility Foreman, be joined in the use of one connecting sewer lateral, provided that the connection conforms in all other respects to the provisions of this chapter and a drawn plan of the joint connection be first submitted to and approved by the City Engineer and/or City Public Utility Foreman.  As a further condition of obtaining such a permit, all buildings and structures shall be owned by the same person.

A sewer lateral is not required for accessory buildings.

(3)	Where two (2) or more occupied buildings are situated on adjoining parcels of land, with or without all parcels being under the same ownership, and where it is physically impossible and/or economically unfeasible to provide separate and independent sewer lateral lines to each occupied building, the City Public Utility Foreman and/or City Engineer may permit the use of a joint/combined sewer lateral service provided the parties have entered into a mutual agreement for the future maintenance, repair, replacement, and billing of the lateral service line.

(4)	Where, in the opinion of the City Engineer and/or City Public Utility Foreman, it is impossible or impractical to connect a building on a single lot to the main sewer except in conjunction with the connection of a building or buildings on other lots, a branched or common connection may be allowed, provided that the connection conforms in all other respects to the provisions of this chapter and a drawn plan of the branched or common connection be first submitted to and approved by the City Engineer and/or City Public Utility Foreman.  A mutual agreement for the future maintenance, repair, replacement and billing of the lateral service line will be required of all involved parties and filed with the City Public Utility Foreman.

(5)	Where two or more structures on separate parcels are connected to a branched or common lateral, each property shall be disconnected from the branched or common lateral and a new separate lateral shall be constructed upon the transfer of title of either property by sale.

(D)	A cleanout approved by the City Engineer and/or City Public Utility Foreman shall be installed and maintained, at the sole expense of the property owner, on all sewer laterals.  The installation of the devices shall be required as follows:

(1)	When building a new structure on property with an existing lateral, or when otherwise proposing to connect a previously unconnected structure to an existing lateral;

(2)	As a condition of approval of any major building remodel project which may affect or impact any current or potential performance of the building’s sanitary wastewater management system.  A major building remodel project is one that is estimated by the city to cost $50,000 or more;

(3)	Prior to the close of escrow when any property connected to the City sanitary wastewater discharge system is transferred via sale or other transfer of ownership by deed, instrument or writing;

(4)	Whenever the city finds that a sewage spill emanating from a lateral has reached public property, including but not limited to a city street or the city storm drain system, or has flowed onto private property owned by another property owner; and

(5)	Whenever the city finds that a sewage spill emanating from a lateral presents a threat to public health, even if it has not flowed across a property line.

(E)	In addition to the general provisions contained in 52.04 (B), back traps or check valves approved by the City Public Utility Foreman and/or the City Engineer may be required to be installed and maintained, at the sole expense of the property owner, on sewer laterals if the following conditions exist:

(1)	On laterals serving properties where the outlet of a trap for a plumbing fixture is installed or located at an elevation which is less than two feet above the rim of the nearest manhole or other sewer access point uphill from the point of connection of the lateral to the public sewer in any new or existing drainage system.  The installation of the devices shall be required under the same circumstances as set forth in subsections (D)(1) through (5) of this section.

(2)	Where an approved back trap or check valve is required, the valve shall be installed in the lateral at the point of lowest elevation of the ground surface along the alignment of the lateral, or at such other location as is permitted by the city, providing that at any such location, the elevation of the ground surface is not less than two feet below the lowest trap served by the building sewer.

(3)	The back trap or check valve shall have cleanouts directly upstream and downstream of the valve.  The cleanouts shall be connected to the lateral by means of wye fittings.  The back trap or check valve shall be accessible from the surface and protected by the use of a precast access box of concrete, heavy-duty plastic, or other material approved by the City Public Utility Foreman and/or the City Engineer.

(4)	The cleanouts shall be positioned at an elevation at least three inches above the ground in order to prevent the obstruction of the vent opening or the inflow of surface water.

(F)	Any owner whose property meets the elevation criteria of this section that has no back trap or check valve, or has a defective or improperly installed back trap or check valve, shall be responsible for all damage that results from the lack of such a device, or the failure of the defective or improperly installed device to prevent such damage.

52.53 SUMP PUMP/FOUNDATION DRAIN TILE REGULATIONS.

Any dwelling, structure, or building that has a sump pump or gravity flow discharge system to remove groundwater from its foundation drain, must have a permanently installed discharge line.

A “permanently installed discharge line” shall be one which provides for year–around discharge capability to either the outside of the dwelling, building, or structure, or is connected to the city storm sewer.  In no event shall the discharge be outletted or allowed to flow across public property including street boulevard areas, or property owned by others.  It shall consist of a solid (or closed) discharge line, without valving or quick connections for altering the path of discharge and, if connected to the city storm sewer line, include a check valve.  It shall not be capable of connection or reconnection to the municipal sanitary sewer system.

Any person, firm, or corporation having a roof surface, groundwater, sump pump, footing tile or swimming pool now connected and/or discharging into the sanitary sewer system shall immediately disconnect and/or remove same.  Any disconnects or openings in the sanitary sewer shall be closed in an effective, workmanlike manner, as approved by the City Engineer, the City Public Utility Foreman, or their designee.

Upon notice that the discharge of clear water on a property is not in compliance with this ordinance, the owner or occupant of the property shall cease from discharging clear water in violation of this ordinance and shall make the necessary repairs and corrections to discharge the clean water in accordance with this ordinance.  Discharge of clear water in compliance with this ordinance shall be completed within 60 days of the date of notice of non-compliance, or as determined by the City Engineer and/or City Public Utility Foreman.  A second inspection of the property will be performed within 60 days following the notice of non-compliance.

52.54  INSPECTION OF SEWER LATERALS.

The purpose of the inspection shall be to confirm that sanitary sewer lateral is fully tight and operational and not allowing inflow, infiltration, roots, debris, etc., to enter as well as to confirm that no sump pump or other prohibited discharge is being conveyed to the lateral.  A certificate of compliance shall be issued by the city upon successful completion of an inspection or following successful repairs of a failing lateral line or sump pump discharge location.  A certificate of compliance shall be valid for ten (10) years from its date of issuance.

Unless the property owner already has a valid certificate of compliance issued by the City for the property, the owner or owner’s representative is required to complete and obtain a certificate of compliance issued by the City for the following circumstances/reasons/conditions:

A.  When building a new structure on property with an existing lateral, or when otherwise proposing to connect a previously unconnected structure to an existing lateral;

B.  As a condition of approval of any major building remodel project which may affect or impact any current or potential performance of the building’s sanitary wastewater management system.  A major building remodel project is one that is estimated by the city to cost $50,000 or more;

C.  Before any developed property is offered for sale, gifted, or transferred, and before the owner or owner’s representative enters into any contract for deed or other transaction changing the party responsible for the property, provided, however that transfers by order of a court having jurisdiction over the property, or transfers between family members made as part of a bona fide estate plan without consideration, or as a transfer out of a probate shall not be included within this section;

D.  Whenever the city finds that a sewage overflow emanating from a lateral has reached public property, including but not limited to a city street or the city storm drain system, or has flowed onto private property owned by another property owner;

E.  Whenever the city finds that a sewage overflow emanating from a lateral presents a threat to public health, even if it has not flowed across a property line.  In the absence of a specific deadline, all inspection and testing work shall be completed within sixty (60) days of notification by the city that such inspection is required.  Existing laterals shall not be used if they are found to be defective by the inspection or if they fail city mandated tests or if they were constructed of materials deemed unacceptable by the City Engineer and/or the City Public Utility Foreman;

F.  As part of its periodic construction and maintenance of sewer mains, the city may discover defective laterals.  The City may order the property owner to conduct an inspection, repair or replacement of any lateral that the city knows or reasonably suspects to be defective; and

G.  In conjunction with the City’s ongoing infrastructure replacement project(s), those properties not possessing a valid certificate of compliance shall be required to obtain a sewer lateral/sump pump inspection within sixty (60) days of notification from the City, and shall be performed by the City at its cost..  If repairs of the sump pump connections and/or the sanitary sewer lateral lines are required, they shall be performed and assessed in conjunction with the infrastructure replacement project(s’) work activities.

Any building may be re-inspected on a yearly basis in conjunction with yearly water meter or other inspections.

All new construction will be required to have a sump pump system inspection performed and be in compliance with the provisions of this Chapter.

52.55 REPAIR OF SEWER LATERALS

- Whenever defective laterals are found, the property owner, at its sole expense, shall repair or replace the lateral.  The City Engineer and/or the City Public Utility Foreman shall determine the extent of the repair required.  More limited repair than complete replacement of the lateral may be permitted at the sole discretion of the City Engineer and/or the City Public Utility Foreman.  The following requirements shall be met:

- A replaced or repaired lateral shall not be covered or backfilled until it has been inspected by a representative of the City;

- All new and repaired laterals must pass an air pressure test as specified by the City Engineer and/or the City Public Utility Foreman; and

- All repaired or replaced laterals shall be brought into compliance with the requirements of this chapter.

B.  In the absence of a specific deadline established by the City Engineer, and/or the City Public Utility Foreman, all repair or replacement work shall be completed within sixty (60) days of notification by the City that such repair or replacement is required.

C.  When a lateral is completely replaced, the property owner is not required to inspect the lateral upon sale of the property for a period of ten (10) years following the date of complete replacement of the lateral.

D.  Roots, grease, or other material which have accumulated in a lateral being cleaned or maintained shall be prevented from entering the sewer main during the maintenance or repair of the lateral.  In the event that material is permitted to enter the main causing or contributing to the cause of a sewage spill, the property owner and/or contractor performing such maintenance work, in addition to any criminal penalties imposed, shall be held civilly liable to the City for any fines or other expenses incurred by the City resulting from the spill.

E.  The City shall have the authority to recover from a property owner the City’s expenses incurred in responding to sewer overflows on private property.  In addition to any actual expenses incurred by the City resulting from an overflow, the City may impose civil administrative penalties against a property owner who fails to perform any act required in this section, which failure results in an overflow reaching public or private property other than the property owner’s property, according to the following schedule:

(1)  Up to $500.00 for the first violation;

(2)	Up to $1,000.00 for the second violation occurring within three years after the first violation; and

- Up to $2,500.00 for each additional violation within a three-year period following two violations.

F.  The city shall have the authority to establish, waive, suspend, or otherwise modify any civil administrative penalty imposed by this section that exceeds the direct costs of the city upon a showing by the property owner of severe financial hardship, or upon showing that the property owner has satisfactorily repaired the lateral to a degree sufficient to ensure avoidance of further violations.

G.  The provisions of this section shall not be construed to require or obligate the city to enter into a reimbursement agreement if, in the sole discretion of the City Council, to so enter into the agreement would not be in the best interests of the city or would be detrimental to the health, safety, or welfare of the city.

52.56  SEWER LATERAL REQUIREMENTS ASSOCIATED WITH BUILDING DEMOLITION

Where buildings are to be demolished, the following actions shall be required:

1.  Determine if the service is to be abandoned or reused in the future; and

2.  Have the service (lateral) line televised if records are not on file showing it to have been televised in the past ten (10) years.  A city official shall review the video recording and records on file with the City to determine the condition of the sewer lateral.  (Typically, the City Public Utility Foreman or City Engineer).

A.  Sewer Lateral to be Abandoned

1.  If the service line is to be abandoned and is in good condition with no apparent inflow and infiltration issues (typically either PVC or cast iron pipe), remove or abandon the service line to the property line.  Expose the sewer lateral at the lot or curb line and seal/cap the open end to the satisfaction of the authorized city official.

2.  If the service line is to be abandoned and is in poor or failing condition and the street is in poor or failing condition, remove or abandon the service line to the sewer main.

3.  If the service line is to be abandoned, and is in poor or failing condition but the street is in fair to good condition, remove or abandon the service line from the building to the property line, or as near as possible to the back of the curb, and do either of the following alternative actions with the remainder of the sewer line to the sewer main:

a.  Dig down to the lateral, exposing the line at the lot line or close to the curb and install a city approved membrane liner through the remaining service stub.  Seal both ends of the liner, and install a seal/cap on the open end at the property or curb line to the satisfaction of the authorized city official; or

b.  Install a city–approved plug in the service line as near the main line pipe as possible.  Fill the remaining service stub with a non-permeable grout or sealant and install a seal/cap on the open end of the pipe at the property or curb line to the satisfaction of authorized city official.

B.  Sewer Lateral to be Reused

1.  If the service line is needed in the future and the service line is in good condition (typically either PVC or cast iron pipe) regardless of the street condition, remove or abandon the service line to the property line.  Dig down to the lateral, exposing the line at the lot line or close to the curb and install a cap, or seal the open end of the pipe at the property or curb line to the satisfaction of the authorized city official.  Provide location (ties- minimum of two) to the city official.

2.  If the service line is needed in the future, but is in poor or failing condition and the street is in poor or failing condition, remove or abandon the service line from the building to the mainline.  Remove the old connection at the main line, and install a new wye and pipe from the main sewer line to the lot line and install a cleanout access at the property line as determined by the authorized city official.

3.  If the service line is needed in the future, but the old line is in poor or failing condition and the street is in good to fair condition, the service line shall be removed or abandoned to the property line or curb.  Install a sewer cleanout at the property line or near the curb.  Connection to the remaining sewer service stub between the sewer cleanout and the main line sewer pipe shall either:

a.  Have a city-approved membrane liner installed and have the line capped at the lot line below the cleanout;  or

b.  Have a city-approved plug installed in the service pipe as near the main line pipe as possible, then filled with a non-permeable, but removable, grout, or sealant.

52.57  WAIVERS

The City Council shall have the power and duty of hearing and deciding requests for waiver from the applicability of the provisions of the chapter 52 where strict enforcement would cause undue hardship because of circumstances unique to the individual property under consideration or cause a safety problem.  This may also include cases where it would not be practical or feasible to correct a clear water discharge problem.

Application for waivers pursuant to the Chapter 52 shall be addressed in writing to the City Administrator.  The applications shall, at a minimum, identify the subject property, the name of the property owner/applicant, and describe in detail specific characteristics of the subject property that create an undue hardship.  Within a reasonable time, the Council shall make its decision on the matter and send a copy of such decision to the applicant by regular mail.  Upon approval of an application for a waiver, a property owner shall be allowed to discharge directly into the sanitary sewer system for a limited time specified in the Council’s written decision and in accordance with the other terms and conditions specified by the council.  The applicant will be required to agree to pay an additional fee for the additional sewer service, along with the regular monthly charge.  Fees for this service will be based on estimated yearly average amounts discharged to the sanitary sewer.

52.58 SURCHARGES AND PENALTIES

A surcharge of $100.00 per month will be added to every sewer bill sent to property owners who have not obtained a required property inspection or who are not in compliance with this Chapter 52.  The surcharge shall be added every month thereafter for properties not complying with this Chapter 52.  All properties found during any inspection to have violated this Chapter 52 will be subject to the $100.00 per month charge for all months between the two most recent inspections, and for each month thereafter until the property owner submits proof to the City that that the property is brought into full compliance.  If the surcharge is not paid, the City reserves the right assess the property owner the unpaid balance pursuant to Minnesota Statute 429.101.  In addition to the $100.00 per month charge, a property owner or other person who is not in compliance with the Chapter 52 may be mailed, by regular mail, a notice that such violation must cease and desist within a time limit established (set) by the Council.  If such violation does not cease and desist within the time limit, the owner of the property or other person in violation Chapter 52 shall be guilty of a misdemeanor.

52.59  SEVERABILITY AND VALIDITY

The provisions of this ordinance are hereby declared to be severable.  If any provision, clause, sentence, or paragraph of this ordinance or the application thereof to any person, establishment, or circumstances be declared by a court of competent jurisdiction to be invalid, such invalidity shall not affect the validity of other provisions or application of this ordinance.

Passed by the City Council of Buhl, Minnesota this _______ day of ____________________, 2025.

_________________________________________

Mayor

Attested:

__________________________________________

City Administrator